Goal:    Use the PmodMAXSONAR module to display the distance to the nearest target.

Resources:

Basys3 Reference Manual

Basys3 Schematics

PmodMAXSONAR Reference Manual

- PmodMAXSONAR pin 2 (RX) must be held high or floating for the module to function

- The VAUX_n side of the Spartan 6 XADC pin must be grounded to make correct measurements

- XADC clock frequency must be >= 4 MHz in order to supply samples

Minimum Requirements:

Use the 7-segment display to show analog readings from the PmodMAXSONAR sensor

Bonus Features:

Produce updates as fast as the sensor allows (49 ms)

Coherently integrate (average) more than 256 samples to stabilize distance measurement

Measure distances beyond 127 inches correctly

Use the LD0-15 LED array to display distance and/or velocity as a bar chart

Implement velocity readout (in units per second)

Calibrate the display to show inches or centimeters accurate to ±0.5 inch (±1.3 cm)

Remove any display digit bleedover, suppress leading zeros, and show negative velocities

Display the number of samples produced per second

….Anything else you think might be cool